# EXPERIMENTAL WORKFLOW

VINDTA + DIC measurement NICO REMKES version 1 23-09-2025

In this document I will describe:

- The different phases of the experiment

- The individual steps and their (estimated) duration

- Relevant parameters

- Considerations

- Possible experimental variations

The following document is a work in progress and should be updated after each experiment to show which experimental variation was used and when. (and crucially if considerations/relevant parameters need to be updated)

There are 3 phases:

- The sample and instrument preparation phase

- The VINDTA phase

- The bobometer/DIC phase

For now the main focus of the experiments will be to look at the effect of acid addition amount and the timing of different steps

Contents

EXPERIMENTAL WORKFLOW	1

Phase 1: Sample and instrument preparation	3

Sample preparation	3

VINDTA preparation	3

BOBOMETER preparation	3

Thoughts	3

Timings	4

Phase 2: Alkalinity measurement using the VINDTA	5

Alkalinity measurement	5

Relevant parameters	6

Time	6

Headspace	7

Thoughts	7

CODING THOUGHTS (discuss with Matthew)	7

Timings	8

Experimental variations VINDTA	9

Experiment set 1	9

Experiment set 2	9

Experiment set 3	9

Experiment set 4	9

Phase 3: DIC measurement using the BOBOMETER	10

DIC measurement	10

Relevant parameters	10

Thoughts	10

Timings	10

Experimental variations DIC	11

Experiment set 1	11

Experiment set 2	11

Experiment set 3 (discuss with Matthew if this really is necessary)	12

## Phase 1: Sample and instrument preparation

The first phase is perhaps the simplest, but nevertheless important to ensure consistency.

In the first phase the sample and the instruments (VINDTA and BOBOMETER) need to be prepared.

### Sample preparation

The sample should be placed in the water bath to equilibrate ??

#### Considerations:

- Ideally the headspace volume should be controlled, draw a line/tape on a specific height and try to always fill to the same level such that any outgassing results in a systematic DIC loss.

- Measure the actual temperature of the water bath! Don’t necessarily trust the setpoint!

#### Questions (For Matthew )

- Cleaning procedure for sample bottom before sampling?

- How long in the water bath?

### VINDTA preparation

The VINDTA needs to be set up such that it can start a titration measurement. Perhaps a junk measurement needs to be run? Add detailed description from notebook later

#### Considerations:

- Keeping track of the stability of the instrument is important. One option to test this would be the speed at which a junk measurement is processed

- Acid ends at 4.2 mL by default, but last recorded point is at 4.05 mL, acid is added physically however, so there’s a data gap….

#### Questions (For Matthew )

- Can we fix the 4.2 mL datapoint? Perhaps the hardcoded delay is causing issues? Or something in the method? Check if previous datasets have the same issue?

- What is the usual VINDTA prep procedure/ running junks sample?

### BOBOMETER preparation

The BOBOMETER needs to be set up such that it can start a DIC measurement. Perhaps a junk measurement needs to be run? Add detailed description from notebook later

#### Considerations:

- Keeping track of the stability of the instrument is important.

#### Questions (For Matthew )

- What is the usual BOBOMETER prep procedure/ running junks sample?

## Thoughts

Perhaps it’s a good idea to always start each measuring day with several 0 acid added measurements as these are quick (limited by BOBOMETER rather than VINDTA so approx 15 minutes each measurement) these should give quick results about drift/stability of the BOBOMETER. For the VINDTA it gives some insight into the stability of the EMF maybe? Running full titrations to get the TA would be time consuming, but could be an option as well.

## Timings

Daystart = only at the start of the day

| Task | Duration | Notes | simultaneous |
| --- | --- | --- | --- |
| Turn on water bath (Daystart) | ???? | Daystart |  |
| Clean sample bottle | ???? |  |  |
| Dry sample bottle? | ???? |  |  |
| Fill sample bottle | ???? |  |  |
| Sample in water bath | ???? |  |  |
| Setup VINDTA (Daystart) | 5-10 min | Reattach pump, gas, electrode caps off, test electrode, flush acid line, setup file naming convention for day |  |
| initialize VINDTA (Daystart) | 25 min | Run a junk????? | YES |
| Setup DIC (daystart) | ??? | Empty waste, fill cell with solutions and salt, close lids (but big one only partly). Connect laptop (my CO2 network and 192.168.4.1) Turn on gas connection, flip switch from cell off to N Connect gas flow (From switch 4) to cell. Setup outflow pump to exhaust Support outflow pump and gas pump with stand. Push big lid down all the way. Twist cell till best transmission, turn knob to get 100% Attack black and red pins. |  |
| Initialize DIC (Daystart) | ???? | Fill stripper with acid, watch for bubbles Run script_initialize? Run a junk? | When Titration starts |
| Entry log (daystart) | 3 min | Write plan for the day in the log at Daystart, |  |
| Visual inspection (daystart) | 2 min | Check both instruments thoroughly |  |
| Prepare Vindta | 1 min | Ensure there is enough rinsing solution for the day, enough acid in the Titrino, Gas line to sample bottle is off, VINDTA ready to attach sample and go |  |
| Prepare DIC |  | Ensure main script is loaded on website, doublecheck all solution levels within the instrument. |  |

In principle at the end of phase one both the VINDTA and BOBOMETER should be ready to go at the click of a button/flip of a switch. Logbooks already filled in, scripts loaded,

# Phase 2: Alkalinity measurement using the VINDTA

Once everything is set up the actual measurement can start. Depending on the desired experiment the VINDTA phase can be very short or relatively long. For a relatively long run perhaps the BOBOMETER can be run in parallel, to minimize downtime. As long as the VINDTA is not done before the BOBOMETER is done processing the previous measurement there should be no problem, but starting the VINDTA a few minutes later can also help. FIGURE OUT EXACT TIMINGS TO DO THIS RELIABLY.

## Alkalinity measurement

The alkalinity measurement is controlled almost entirely by the software, the limited human input is done in the beginning, adding the sample, pressurizing it and using the software to start the measurement with the desired parameters and a predetermined filename. There are several parameters that can be changed however, and the titration process itself is the focus of this study so initially all parameters need to be studied closely to determine their impacts on the Alkalinity as well as the DIC.

## Relevant parameters

Time

Timings show up in several steps of this process and are likely to have an impact on the final result. Obtaining a better understanding of the process helps in improving the experimental protocol. Where there are two extremes.

QUICK

Doing things as quickly as possible reduces the time the system has to equilibrate, which could reduce the impact of degassing of CO2. It could also be more difficult to reproduce as the relevant timescales might chance to seconds.

SLOW

Doing things slowly on purpose gives the system more time to equilibrate, in which it drifts further from the theoretical expectation in an ideal system without any degassing. With the right experiments however perhaps this difference can be predicted/modelled more accurately making the measurements more reproducible.

The steps where time plays a role are:

- How long the sample has been heated before the VINDTA begins drawing it

- How quickly the VINDTA transfers the sample to the cell

- The time it takes to add x amount of acid

- The time in between acid additions

Time in between additions is variable according to the pattern in the following plot. (keep updated with newer data!)

)

Headspace

In various parts of the process the sample is in contact with air, where there might be CO2 exchange. Any loss or gain in CO2 during this process will show up in the final DIC. Knowing in which part of the process there is gain or loss could help in estimating the loss or quantifying it to a certain extent.

There are various headspaces which influence the experiment, some might result in DIC gains and most will result in DIC loss. From theoretical models the pCO2 reaches extremely high values at lower pH values from Henry’s law.

In that sense the time element becomes more crucial at lower pH. As there the headspace is suddenly capable of storing much larger amounts of CO2.

| Headspace | Size (approx.?) | Gain/Loss | BIG? |
| --- | --- | --- | --- |
| Sample bottle | Depends on filling height 20 mL? | Depends on previous equilibration, could be a gain? | Small |
| Pipette cell | Few mL | Depends on previous equilibration, could be a gain? | Small |
| Titration cell | Freely connected to lab through | Loss | BIG (time dependent) |

## Thoughts

The titration process is presumably always lagging behind equilibrium, this makes it difficult to determine whether a change is due to the system catching up to equilibrium from previous input or that the new input (one extra bit of acid added) is resulting in the change. Always be aware of this!

## CODING THOUGHTS (discuss with Matthew)

Current file conversion relies on a linspace in which the addition of acid is fixed numerically to be 0.15 mL each increment. This is not a robust approach towards the future, and should be turned into a variable. Need to have a think of how to do batch processing reliably when datasets have mixed acid addition rates. Personally I think adding a tkinter window thing might be an easy solution. Assuming making the bak files is done each day there should be at most 20 files, where the values can be initialized to a set variable (0.15) and changed if required.

## Timings

Create detailed timings table of all steps when running experiment

| Task | Duration | Notes | simultaneous |
| --- | --- | --- | --- |
| Inspect instrument for leaks/abnormalities | 30 seconds | Look for fluid drops anywhere, check if cables look proper |  |
| Create bottle file name if not already in edit sample list | 20 seconds | Write convention here for easier checking, check what convention is used for samples |  |
| Set titration parameters in software | 20 seconds | write in log immediately! |  |
| Connect sample bottle to system | 15 seconds | Squeeze tightly |  |
| Turn on gas to sample | 5 seconds |  |  |
| Waiting phase? | 0 seconds ???? | Perhaps nice to incorporate a fixed amount of waiting time at some points |  |
| Start measurement using bottle button in software |  | Take note that cell gets rinsed and pipette overflows during filling stage. Record bottle press time |  |
| Pipette filling | ???? | Perhaps nice to time this? |  |
| Pipette draining into cell/full cell |  | Record time |  |
| First drop of acid added |  | Record time |  |
| Drops of acid added |  | Record time, not required each measurement |  |
| Final drop of acid added |  | Record time |  |

## Experimental variations VINDTA

In the titration process there is a lot of potential variations.

### Experiment set 1

Stop the titration at various points along it’s normal procedure. In a single day it should be feasible to run a number of measurements. If measuring from 0 to 4.2 mL of acid. Taking measurements at intervals of 0.6 mL or 0.3 mL of acid added (While adding 0.15 mL acid during the titration as per usual) when more proficient with the VINDTA perhaps it is feasible to do a full dataset with 0.15 mL increments in order for this to work, interchange long and short measurements or use exact timings established previously to eliminate downtime of the DIC for low acid added. E.g measurement 0.15 takes 5 minutes in the vindta, but 10-12 minutes to analyse in the DIC. Start measurement of 0.3 ml, which takes 7 minutes 5 minutes after the DIC one starts to ensure the DIC can work continuously (but there is no excessive outgassing time for the sample in the vindta, Tricky take some safety factor of a few minutes)

### Experiment set 2

Don’t add acids in increments, but simply add the desired dose immediately all at once. This gives the sample much less time to equilibrate, and especially at higher acid dosages it could reveal the time dependent effects. Use a range of 0-4.2 mL again with specific intervals (corresponding to experiment 1)

### Experiment set 3

Add acid in higher/lower increment dosages. Depending on outcomes of experiment set 2, changing the amount of acid added in each step is just a way of manipulating the time scales involved in the experiment. Adding a total of 4.2 mL of acid in 1 minute, 5 minutes, 10 minutes, 30 minutes or 60 minutes etc. will affect the final DIC and gives some (exponentially decaying?) DIC loss function which can then be used to test on different datasets.

### Experiment set 4

Vary the time in between phase 2 and 3 deliberately. From experiment set 1-3 find maybe a nice dosage of acid and then wait x amount of time before measuring it with the bobometer. Somewhat similar to experiment set 3 however in that one the previous steps get more time to equilibrate, while in this experiment the final step gets more time to equilibrate. There could be a significant difference. Potential result could be that: at relatively high pH (CO2 concentrations small) there is no significant effect of waiting (system is close to equilibrium) while at lower pH there is a significant amount of outgassing happening (should see some change occurring along the acid added curve?)

# Phase 3: DIC measurement using the BOBOMETER

The final phase is measuring the DIC content of the sample. This can be done at various stages in the regular titration process, or in a deliberately modified titration process.

## DIC measurement

Describe the process, add some sketch with valves?

## Relevant parameters

Volume and flow rate:

The syringe that draws up the volume can be controlled in the amount and the speed in which it sucks up the sample from the VINDTA. Taking it in more quickly gives the sample less time to degas in the cell. Taking in less sample overall has the same effect (at the same plunger speed).

A smaller volume might however produce less accurate results (as there is less CO2 to work with.)

## Thoughts

## Timings

| Task | Duration | Notes | simultaneous |
| --- | --- | --- | --- |
| Inspect instrument for leaks/abnormalities | 30 seconds | Look for fluid drops anywhere, check if cables look proper |  |
| Connect intake hose to VINDTA | 10 seconds | Place gently into cell, disturb headspace as little as possible |  |
| Start scripts_main | 5 seconds | Flip switch to start script |  |
|  | ???? | Rinsing phase |  |
|  | ???? | Filling syringe phase (create TABLE for filling params and corresponding time!) |  |
|  | ???? | Emptying syringe, record time! |  |
|  | ???? | Bubbling/measuring phase |  |
|  |  | Measurement done after xx:xx |  |
| Save the data | 60 seconds | Press save, and log last value of integrated current |  |

## Experimental variations DIC

### Experiment set 1

The current protocol draws up a small amount of sample to rinse the tubes (twice???). During this time the sample in the VINDTA cell is still degassing. Shortening or removing this step altogether could result in HIGHER DIC-values, further away from equilibrium. Add lower pH there is a (suspected) higher amount of degassing happening, so there could be a significant difference in drawing up the sample immediately or first drawing a small amount of it.

There are various things to test:

- Run it in normal configuration

- Skip the rinsing step and draw the sample immediately

- Overdraw immediately (take on the first pull enough for rinsing and the measurement in one go)

We can test this at 0,1,2,3 and 4 mL of acid to see if there are any effects. Requires 15 experiments so should be just about feasible in a day.

#### Thoughts / consideration

I’m intuitively thinking that option 3 for the filling is the best option by default. There are definitive upsides and no theoretical downsides I expect. (if we assume the plunger/syringe works perfectly) In that sense it would not make that much sense to experiment with this as this should be a slam dunk improvement. Perhaps it’s good to quantify by how much though?

### Experiment set 2

Another way to reduce the time the sample can degas is by increasing the plunger speed. Higher plunger speed should mean higher DIC-values. It should be noted that for this experiment the timescale of the plunger extraction might be similar to experimental variations in the duration of the titration. THIS NEEDS TO BE CHECKED VERY CAREFULLY, otherwise there might be correlation but not causation between DIC and plunger speed.

Once again, some measurements will be more time sensitive than others, so it’s best to ‘stress test’ this with a high rate of degassing type measurement. Perhaps one where acid is dumped in a large quantity instantly. E.g 3 mL at once, and then measure after x-seconds of stirring with 4 different pump speeds to see if there is any difference. It’s a bit tricky to get a trend from different measurements maybe due to their innate variability?

For Matthew, what is the upper limit (safety!) of the pump? 4X times faster than current? Or even 10x? the speed for pushing the sample into stripper is not important I think?

### Experiment set 3 (discuss with Matthew if this really is necessary)

Reducing the syringe volume.

Drawing 25 mL of the sample is quite a lot, perhaps this can be reduced without compromising the accuracy of the DIC measurement.

#### Thoughts / consideration

As long as the DIC leads to a substantial pH change in the cell there should be no problem with having a lower volume. It should be noted however that DIC-values also change one order of magnitude depending on where in the titration or after how long a waiting period you take before taking the DIC. In that sense testing it should be done in the extreme limit, what is the lowest DIC we want to be able to measure? 100 umol? 50? Maybe first run some experiment to see ‘naturally’ occurring DIC values in the process

#### Critical thoughts

Changing too many things seems unwise, the benefit of drawing a smaller amount is limited to experiments where you track degassing over time for a fixed acid added, as you could draw from the same titrated sample several times.

The downside is that you might run into problems where in some cases you used different volumes and the comparisons/continuity of the dataset becomes more questionable. Ensuring compatibility with past and future datasets seems to me more important. Drastically reducing the volume (to below 10mL?) seems therefore to be unwise.

Simple experiment to test this:

- Draw sample twice from two titrations, after 0,10,20,30 minutes (or however long the DIC Takes)

- Draw sample continuously from a single titration (at small volume +- 10mL) 0,10,20,30 minute